# Post-Quantum Broadcast Authentication and Verifiable Delay Functions for IEC 61850 Industrial Control Systems

Abstract

Industrial control systems (ICS) governing electrical substations, chemical plants, and critical manufacturing rely on the IEC 61850 protocol suite for real-time multicast communication. The Generic Object Oriented Substation Event (GOOSE) and Sampled Values (SV) protocols transmit protection and measurement data with sub-millisecond latency requirements, yet their native multicast model provides no source authentication, leaving them vulnerable to spoofing and replay attacks. Simultaneously, safety instrumented systems (SIS) depend on software-enforced timing delays—emergency shutdown cooldowns, interlock holds, and purge cycles—that an attacker with sufficient computational resources can bypass by fast-forwarding timer logic.

This paper presents two complementary constructions that address these threats in a post-quantum setting. First, we introduce TESLA++, an extension of the Timed Efficient Stream Loss-tolerant Authentication (TESLA) protocol that replaces classical primitives with quantum-resistant alternatives: SHAKE256-based one-way hash chains, HMAC-SHAKE256 message authentication codes, and ML-DSA-65 digital signatures for chain commitment bootstrapping. We specify three protocol profiles—GOOSE (10 ms intervals, <200 μs authentication), SV (250 μs intervals, <50 μs MAC computation), and MMS (100 ms intervals)—each calibrated to IEC 61850 timing constraints. Second, we propose a Verifiable Delay Function (VDF) construction based on iterated SHA3-256 hashing that produces non-parallelizable, efficiently verifiable timing proofs for safety-critical operations. Five safety profiles are defined covering SIL 2 and SIL 3 requirements with delays ranging from 10 to 120 seconds, accompanied by ML-DSA-65 signed attestations for regulatory audit trails. Security analysis demonstrates that TESLA++ achieves source authentication under the one-wayness of SHAKE256 and unforgeability of ML-DSA-65, while the VDF construction guarantees sequentiality under the random oracle model. Performance evaluation on representative industrial hardware shows MAC computation latencies of 38 μs for SV and 142 μs for GOOSE, with VDF verification completing in under 0.8 ms regardless of the original computation duration.

Post-quantum cryptography, IEC 61850, TESLA, broadcast authentication, verifiable delay functions, SCADA security, functional safety, ICS

# Introduction

The digitalization of electrical substations, process plants, and manufacturing facilities has placed the IEC 61850 communication standard at the center of modern industrial automation. IEC 61850 defines three principal service mappings—GOOSE for event-driven protection signaling, SV for continuous measurement streaming at rates up to 4 kHz, and MMS for supervisory control—that together form the communication backbone of substation automation systems (SAS) (IEC 61850-1 2013, IEC 61850-7-2 2010). These protocols operate over switched Ethernet with strict real-time constraints: a GOOSE trip command must propagate across a protection zone in under 4 ms, while an SV sample must arrive within a 250 μs window (IEC 61850-9-2 2011).

The multicast nature of GOOSE and SV communication introduces a fundamental security challenge. Because messages are broadcast to all subscribers on a LAN segment, any device with network access can inject forged frames. Demonstrated attacks include GOOSE spoofing that triggers false circuit-breaker trips (Hoyos et al. 2012), SV injection that corrupts phasor measurements (Hong et al. 2014), and replay attacks that exploit the absence of per-message authentication. The 2015 attack on the Ukrainian power grid underscored the real-world consequences of inadequate ICS security (Case 2016).

IEC 62351-6 specifies security extensions for IEC 61850, including message authentication codes (MACs) for GOOSE and SV frames (IEC 62351-6 2020). However, the current specification relies on HMAC-SHA-256 with symmetric key distribution, offering no mechanism for source authentication in multicast settings—any subscriber holding the group key can forge messages attributed to any sender. Furthermore, all cryptographic primitives specified in IEC 62351 are vulnerable to quantum computing attacks. Shor’s algorithm can break the RSA and ECDSA signatures used for key management, while Grover’s algorithm reduces the effective security of symmetric primitives (Bernstein and Lange 2017; Mosca 2018).

A second, orthogonal concern arises in safety instrumented systems governed by IEC 61508 and IEC 61511 (IEC 61508 2010, IEC 61511 2016). These systems enforce mandatory timing delays—emergency shutdown cooldowns, interlock holds, chemical reaction holds, and purge cycles—that prevent premature restart of hazardous equipment. When these delays are implemented purely in software, an attacker who compromises the safety controller can bypass the timer logic entirely, creating conditions for catastrophic physical harm. No existing ICS security standard provides a cryptographic mechanism to guarantee that a minimum physical time has elapsed.

## Contributions

This paper makes the following contributions:

- TESLA++ Protocol. We extend the TESLA broadcast authentication protocol (Perrig et al. 2000, 2002) with post-quantum primitives, replacing SHA-1/HMAC-SHA-1 with SHAKE256/HMAC-SHAKE256 and adding ML-DSA-65 chain commitment signatures. We define three optimized profiles for GOOSE, SV, and MMS traffic with demonstrated compliance to IEC 61850 timing budgets.

- VDF-Based Safety Timing Proofs. We propose a verifiable delay function construction based on iterated SHA3-256 that produces efficiently verifiable proofs of elapsed time. We define five safety profiles aligned with IEC 61508 SIL levels and present a hardware calibration methodology that maps iteration counts to physical delay durations.

- Security Analysis. We provide formal security arguments for both constructions: source authentication for TESLA++ under standard hash function assumptions, and sequentiality for the VDF under the random oracle model.

- Performance Evaluation. We present comprehensive benchmarks demonstrating that TESLA++ meets all IEC 61850 timing constraints and that VDF verification completes in sub-millisecond time regardless of the original delay duration.

The remainder of this paper is organized as follows. Section 2 provides background on IEC 61850, TESLA, VDFs, and relevant safety standards. Section 3 defines the system model and threat model. Section 4 presents the TESLA++ construction. Section 5 describes the VDF-based timing proofs. Section 6 provides security analysis. Section 7 reports performance results. Section 8 surveys related work, and Section 9 concludes.

# Background

## IEC 61850 Communication Architecture

IEC 61850 organizes substation communication into three logical levels interconnected by a station bus and a process bus (IEC 61850-1 2013). At the process level, merging units digitize current and voltage transformer outputs into SV streams transmitted at 4 kHz (4000 samples per second, one sample every 250 μs) over multicast Ethernet (IEC 61850-9-2 2011). At the bay level, intelligent electronic devices (IEDs) exchange protection signals via GOOSE multicast messages with a transfer-time budget of 3–4 ms for critical trip commands (IEC 61850-8-1 2011). At the station level, MMS provides client-server communication for supervisory control with relaxed timing on the order of hundreds of milliseconds.

GOOSE Protocol. Generic Object Oriented Substation Events carry binary status changes (e.g., breaker position, protection relay trips) as ASN.1-encoded PDUs directly over Ethernet (EtherType 0x88B8). Upon a state change, the publisher retransmits the GOOSE frame with exponentially increasing retransmission intervals (e.g., 2, 4, 8, …ms) to ensure delivery despite frame loss. Each GOOSE message includes a state number (StNum) and sequence number (SqNum) but no cryptographic authentication.

Sampled Values Protocol. SV frames carry digitized analog measurements (currents, voltages, power quality data) as a continuous stream. At 4 kHz with 8 channels of 32-bit samples, the raw data rate reaches approximately 1 Mbit/s per stream. The tight timing requires that any authentication overhead remain below 50 μs per sample to avoid violating the 250 μs sample interval.

MMS Protocol. The Manufacturing Message Specification provides confirmed services over TCP/IP for reading/writing data objects, reporting, and control operations. Timing requirements are on the order of 100 ms to seconds, permitting more computationally expensive security mechanisms.

## TESLA Broadcast Authentication

The Timed Efficient Stream Loss-tolerant Authentication (TESLA) protocol, introduced by Perrig et al. (Perrig et al. 2000, 2002), provides asymmetric-like source authentication for multicast streams using only symmetric cryptographic primitives. The core idea exploits delayed key disclosure combined with one-way hash chains.

Hash Chain Construction. The sender generates a one-way hash chain {K0,K1,…,Kn} where Kn is a randomly chosen seed and Ki=H(Ki+1) for i=n−1,…,0. The anchor K0 is authenticated via a digital signature and distributed to all receivers.

Authentication. Time is divided into intervals of duration Δ. During interval j, the sender authenticates each message m by appending MAC(Kj,j∥m). The key Kj is not yet known to receivers at this point.

Disclosure. After a delay of d intervals, the sender discloses Kj. Receivers verify the disclosed key by checking Hj(Kj)=K0 (walking the hash chain down to the anchor) and then verify the buffered messages’ MACs using Kj.

Security. The one-way property of H ensures that receivers cannot compute future keys from disclosed past keys. Since Kj is unknown to receivers at the time the message is sent, only the sender (who pre-computed the full chain) can produce a valid MAC, achieving source authentication.

## Verifiable Delay Functions

A Verifiable Delay Function (VDF) is a function f:X→Y that requires a prescribed number of sequential steps to evaluate, yet whose output can be efficiently verified (Boneh et al. 2018). Formally, a VDF satisfies three properties:

Definition 1 (VDF Properties).  A VDF with parameter T provides:

- Sequentiality: Any algorithm computing f(x) requires at least T sequential steps, even with polynomial parallelism.

- Efficient verification: Given x, y=f(x), and a proof π, verification runs in time O(polylog(T)).

- Uniqueness: For every input x, there exists exactly one valid output y.

Wesolowski (Wesolowski 2019) and Pietrzak (Pietrzak 2019) proposed efficient VDF constructions based on repeated squaring in groups of unknown order. In our setting, we construct a simpler VDF from iterated hash function evaluation, which is inherently sequential due to the data dependency between consecutive hash invocations.

## IEC 62351-6 Security

IEC 62351-6 (IEC 62351-6 2020) defines security mechanisms for IEC 61850 profiles. For GOOSE and SV, it specifies optional HMAC-based authentication appended to frames. However, the standard relies on pre-shared symmetric keys distributed via IEC 62351-9 Group Domain of Interpretation (GDOI), which provides group-level authentication but not source-level authentication within the group. All group members share the same MAC key, meaning any compromised IED can forge messages attributed to any other IED.

## Safety Integrity Levels

IEC 61508 (IEC 61508 2010) defines four Safety Integrity Levels (SIL 1 through SIL 4) that specify target failure rates for safety functions. SIL 3 requires a probability of dangerous failure on demand between 10−4 and 10−3. Safety instrumented systems (SIS) conforming to IEC 61511 (IEC 61511 2016) implement safety functions including emergency shutdowns, pressure relief, and interlock management, all of which incorporate mandatory timing delays to prevent hazardous premature restarts.

# System Model and Threat Model

## Substation Architecture

We consider a typical IEC 61850-based substation with the following components:

- Merging Units (MUs): Digitize analog CT/VT outputs and publish SV streams at 4 kHz over the process bus.

- Protection IEDs: Subscribe to SV streams and publish GOOSE trip commands. Each IED protects a bay (feeder, transformer, busbar).

- Bay Controllers: Execute control sequences and publish GOOSE status messages.

- Station Gateway: Provides MMS access to the SCADA master and engineering workstations.

- Safety Controller: Implements SIS logic including emergency shutdowns, interlocks, and purge cycles per IEC 61511.

Communication occurs over a switched Ethernet LAN with VLAN segmentation. GOOSE and SV traffic uses Layer 2 multicast; MMS traffic uses TCP/IP. The process bus carries SV traffic with a maximum end-to-end latency budget of 250 μs. The station bus carries GOOSE traffic with a maximum transfer time of 4 ms for Type 1A (trip) messages.

## Communication Requirements

Table 1 summarizes the timing constraints that any authentication mechanism must satisfy.

IEC 61850 Timing Requirements and Authentication Budgets

| Protocol | Transfer | Auth. | Sample | Priority |
| --- | --- | --- | --- | --- |
|  | Time | Budget | Rate |  |
| GOOSE Trip | <4 ms | <200 μs | Event | Critical |
| GOOSE Status | <10 ms | <500 μs | Event | High |
| SV (4 kHz) | <250 μs | <50 μs | 4000/s | Critical |
| MMS Control | <500 ms | <1 ms | On-demand | Normal |

## Threat Model

We consider an adversary A with the following capabilities:

- Network access: A can observe, inject, modify, and replay any frame on the substation LAN. This models a compromised engineering workstation, rogue device, or MITM position on the Ethernet bus.

- Computational power: A has access to polynomial-time classical computation and, in the quantum threat model, a cryptographically relevant quantum computer (CRQC) capable of running Shor’s algorithm.

- Timing manipulation: For safety-critical systems, A may attempt to fast-forward software timers by manipulating NTP sources, reprogramming PLC logic, or exploiting race conditions in safety controller firmware.

- Key compromise: A may compromise one or more group keys used for IEC 62351-6 HMAC authentication, allowing forgery of messages within the compromised group.

The adversary’s goal is either (a) to forge a GOOSE/SV message that passes authentication (spoofing), or (b) to produce a VDF proof without waiting the required physical delay (timer bypass). We assume A cannot compromise the sender’s long-term ML-DSA signing key or physically tamper with the sender’s hardware.

# TESLA++ Post-Quantum Broadcast Authentication

We now present TESLA++, our post-quantum extension of the TESLA broadcast authentication protocol optimized for IEC 61850 multicast environments.

## Design Rationale

Classical TESLA (Perrig et al. 2000) uses SHA-1 for hash chain construction and HMAC-SHA-1 for message authentication. Both are considered insecure: SHA-1 has known collision attacks, and HMAC-SHA-1 provides only 80-bit security against quantum pre-image attacks via Grover’s algorithm. Our design replaces all classical primitives with quantum-resistant alternatives while preserving the fundamental TESLA security model.

The key design decisions are:

- SHAKE256 hash chains: SHAKE256 (Kelsey et al. 2016) is an extendable-output function (XOF) from the SHA-3 family with adjustable output length. We use 256-bit output, providing 128-bit quantum security against pre-image attacks.

- HMAC-SHAKE256 MACs: Per-message authentication uses HMAC with SHAKE256 as the underlying hash, providing quantum-safe integrity.

- ML-DSA-65 commitments: The initial chain anchor is signed with ML-DSA-65 (National Institute of Standards and Technology 2024) (NIST FIPS 204), a lattice-based signature scheme providing NIST Security Level 3 (128-bit quantum security).

- Protocol-specific profiles: Rather than a one-size-fits-all configuration, we define separate profiles for GOOSE, SV, and MMS that balance security strength, bandwidth overhead, and latency.

## Hash Chain Construction

Definition 2 (TESLA++ Hash Chain).  Given a security parameter λ=256 and chain length n, the TESLA++ hash chain is the sequence {K0,K1,…,Kn} where: Kn&←${0,1}256Ki&=SHAKE256​(Ki+1)|256 for i=n−1,…,0 The value K0 is the anchor (publicly committed), and Kn is the seed (secret).

The chain is pre-computed by the sender and stored in memory. Keys are consumed in forward order: K1,K2,…,Kn. The anchor K0 is never used as an authentication key; it serves only as a trust root for chain verification.

Checkpoints. For chains of length n≥10,000, verifying a disclosed key by walking the entire chain to the anchor is prohibitively expensive. We introduce periodic checkpoints: every ⌊n/128⌋ iterations, the sender stores an intermediate hash value. This produces 128 checkpoints that allow verification in O(n/128) hash operations. Each checkpoint is accompanied by a Wesolowski-style proof of approximately 256 bytes.

## Chain Commitment and Bootstrap

Before transmitting any authenticated messages, the sender creates and broadcasts a chain commitment:

Chain anchor K0, chain length n, sender ID, ML-DSA keypair (sk,pk) Signed commitment C cid←${0,1}128 t0←CurrentTime() data←cid∥K0∥n∥t0∥Δ∥d∥sender_id σ←ML-DSA-65​.Sign(sk,data) C←(cid,K0,n,t0,Δ,d,sender_id,σ) C

The commitment C binds the sender’s identity to the chain anchor K0, the interval duration Δ, and the disclosure delay d. The ML-DSA-65 signature σ ensures that only the legitimate sender can create valid commitments, providing the asymmetric trust bootstrapping that distinguishes TESLA from pure symmetric MAC schemes.

## Per-Message Authentication

During interval j, the sender authenticates each message m by computing a truncated HMAC:

tagj(m)=HMAC-SHAKE256​(Kj, j∥m)|τ

where τ is the truncation length in bytes (protocol-dependent). The authenticated message is transmitted as:

wire(m,j)=cid∥j∥|tag|∥tagj(m)∥m

Algorithm [alg:authenticate] details the per-message authentication procedure on the sender side.

Payload m, current interval j, chain key Kj, config P Authenticated message M mac_input←Encode(j)∥m full_mac←HMAC-SHAKE256​(Kj,mac_input) tag←full_mac[0:P.τ] M←(cid, j, tag, m, timestamp) M

## Key Disclosure and Verification

After a delay of d intervals, the sender discloses spent keys in batches. Batch disclosure amortizes bandwidth overhead, which is particularly important for high-rate SV streams.

Current interval jcur, delay d, batch size b, chain {Ki} Key disclosure message D jsafe←jcur−d jstart←max(1, jsafe−b+1) keys←[Kjstart, Kjstart+1, …, Kjsafe] D←(cid, jstart, keys) D

Upon receiving a key disclosure, the receiver verifies each disclosed key by walking the hash chain to a known-good key (the anchor K0 or a previously verified key):

Disclosed key Kj, interval j, anchor K0, verified up to jv Validity flag Ktarget←K0 jtarget←0 Ktarget←Kj′; jtarget←j′ steps←j−jtarget h←Kj h←SHAKE256​(h)|256 h=?Ktarget

Once a disclosed key Kj is verified, the receiver retrieves all buffered messages from interval j and verifies their MACs:

valid(M)⇔tagj(M.m)=?M.tag

## Protocol Profiles

Table 2 summarizes the three TESLA++ protocol profiles optimized for IEC 61850 traffic types.

TESLA++ Protocol Profiles for IEC 61850

| Parameter | GOOSE | SV | MMS |
| --- | --- | --- | --- |
| Chain length n | 10 000 | 100 000 | 5 000 |
| Interval Δ | 10 ms | 250 μs | 100 ms |
| MAC truncation τ | 16 B | 8 B | 32 B |
| Disclosure delay d | 2 | 1 | 3 |
| Auth. latency target | <200 μs | <50 μs | <1 ms |
| Batch size b | 5 | 20 | 1 |
| Chain epoch | 100 s | 25 s | 500 s |

GOOSE Profile. With 10,000 keys and 10 ms intervals, each chain epoch lasts 100 seconds. A 16-byte (128-bit) truncated MAC provides sufficient security for the protection signaling use case while adding only 16 bytes of overhead per frame. The disclosure delay of d=2 intervals (20 ms) balances verification latency against replay protection. Batch disclosure of 5 keys per message reduces bandwidth overhead on the station bus.

SV Profile. The SV profile addresses the most demanding timing constraints: 4,000 samples per second with a 250 μs interval. An 8-byte (64-bit) MAC is used to minimize frame overhead on the high-throughput process bus. The disclosure delay is minimized to d=1 (250 μs) because SV samples are time-critical measurements where delayed verification is acceptable but must occur within milliseconds. The chain length of 100,000 provides a 25-second epoch at 4 kHz, and aggressive batching of 20 keys per disclosure reduces per-disclosure overhead.

MMS Profile. The relaxed timing of MMS permits a full 32-byte (256-bit) MAC for maximum security. A longer disclosure delay of d=3 intervals (300 ms) is acceptable given the non-real-time nature of supervisory operations. Single-key disclosure (b=1) simplifies the protocol for the lower-rate MMS traffic.

## Chain Rotation

When the number of remaining keys in a chain falls below a threshold (100 keys), the sender initiates chain rotation:

- Generate a new hash chain with a fresh random seed.

- Sign a new commitment C′ with ML-DSA-65.

- Broadcast C′ while continuing to use the old chain.

- After receivers acknowledge C′, seamlessly transition to the new chain.

- Continue disclosing keys from the old chain until all pending intervals are resolved.

This overlap period ensures zero message drops during chain handover. The new commitment includes a reference to the old chain ID to allow receivers to associate the rotation.

# Verifiable Delay Functions for Safety-Critical Timing

## Motivation

Safety instrumented systems enforce mandatory timing delays to prevent hazardous premature operations. For example, after an emergency shutdown (ESD) of a reactor, IEC 61511 requires a cooldown period before restart is permitted. If the safety controller’s timer logic is implemented purely in software, an attacker who gains control of the controller can simply skip the delay by manipulating clock values or fast-forwarding the timer state.

A VDF provides a cryptographic guarantee that a minimum amount of sequential computation—and therefore physical time—has elapsed. By tying the delay proof to the actual execution of a sequential hash chain, the VDF ensures that no amount of parallel computation can produce the proof faster than the calibrated delay.

## VDF Construction

Our construction uses iterated SHA3-256 evaluation, which is inherently sequential because each hash depends on the output of the previous one.

Definition 3 (Iterated-Hash VDF).  Given input x∈{0,1}* and iteration parameter T∈N: h0&=SHA3-256​(x)hi&=SHA3-256​(hi−1) for i=1,…,T The VDF output is y=hT with proof π constructed from periodic checkpoints.

The sequential nature follows directly from the data dependency: hi cannot be computed without first obtaining hi−1. Assuming SHA3-256 behaves as a random oracle, there is no shortcut to computing hT other than performing all T sequential hash evaluations.

## Checkpoint Proof Generation

To enable efficient verification, the computation records checkpoints at regular intervals. Specifically, every ⌊T/128⌋ iterations, the intermediate hash value is stored:

Input x, iteration count T Result y, proof π δ←⌊T/128⌋ h←SHA3-256​(x) checkpoints←[] h←SHA3-256​(h) checkpoints.append(h) y←h binding←SHA3-256​(x∥y∥checkpoints) π←|checkpoints|∥checkpoints∥binding (y,π)

The proof structure consists of:

- Checkpoint count: 2 bytes (big-endian unsigned short)

- Checkpoint hashes: 32×N bytes (each checkpoint is a 32-byte SHA3-256 digest)

- Binding hash: 32 bytes (SHA3-256​(input∥result∥checkpoints))

For 128 checkpoints, the total proof size is 2+128×32+32=4,130 bytes.

## Verification

Verification checks the proof structure, the binding hash, and optionally spot-checks segments of the hash chain between checkpoints:

Result y, proof π, config V, input x, timing metadata Validity flag and reason Parse π as (N,{c1,…,cN},binding) (false, “checkpoint count mismatch”) expected←SHA3-256​(x∥y∥c1∥⋯∥cN) (false, “binding hash mismatch”) tmin←V.delay×(1−V.tolerance/100) (false, “timing violation”) (true, null)

The verification procedure runs in O(1) time (constant with respect to T) when relying on the binding hash, or in O(T/128) time if spot-checking individual segments. In either case, verification completes in under 1 ms, compared to the seconds-to-minutes computation time of the VDF itself.

## Safety Profiles

Table 3 defines five VDF safety profiles aligned with IEC 61508 SIL levels and common industrial safety operations.

VDF Safety Profiles for Industrial Operations

| Safety Operation | Delay | Iters. | SIL | Tol. |
| --- | --- | --- | --- | --- |
| ESD Cooldown | 30 s | 30M | 3 | 5% |
| Interlock Release | 10 s | 10M | 2 | 5% |
| Chemical Hold | 60 s | 60M | 3 | 5% |
| Pressure Equal. | 15 s | 15M | 2 | 5% |
| Purge Cycle | 120 s | 120M | 3 | 5% |

Each profile specifies the required physical delay, the corresponding iteration count (calibrated for hardware performing approximately 106 SHA3-256 evaluations per second), the applicable SIL level, and the acceptable timing tolerance. The iteration count is related to the physical delay by T=R×tdelay, where R is the calibrated hash rate of the target hardware.

## Hardware Calibration

The mapping between iteration count T and physical delay t depends on the SHA3-256 throughput of the target hardware. We define a calibration procedure:

Sample iteration count Ts=100,000 Calibrated hash rate R (hashes/second) h←${0,1}256 tstart←PrecisionTimer() h←SHA3-256​(h) telapsed←PrecisionTimer()−tstart R←Ts/telapsed R

Typical calibrated rates range from 106 to 107 SHA3-256 operations per second, depending on the processor. The calibration must be performed on the specific safety controller hardware that will execute the VDF, and the rate should be validated periodically to account for thermal throttling and hardware aging. An appropriate safety margin (e.g., using the lower bound of observed rates) ensures that the actual delay always meets or exceeds the required minimum.

## Delay Attestation

Upon successful VDF computation and verification, the system produces a signed delay attestation for the regulatory audit trail:

A=ML-DSA-65​.Sign(sk, &equip_id∥op_id∥SIL&∥trequired∥tactual∥y∥π)

The attestation binds the equipment identifier, operator identifier, SIL level, required and actual delay durations, VDF result, and proof into a single ML-DSA-65 signed document. This provides non-repudiation for regulatory compliance with IEC 61508 record-keeping requirements and supports post-incident forensic analysis.

# Security Analysis

## TESLA++ Source Authentication

Theorem 4 (TESLA++ Source Authentication).  Under the assumption that SHAKE256​ is a secure one-way function and ML-DSA-65​ is an existentially unforgeable signature scheme, TESLA++ provides source authentication for multicast messages. Specifically, no probabilistic polynomial-time adversary A can produce a valid authenticated message M* for any interval j for which the key Kj has not yet been disclosed, except with negligible probability.

Proof. The proof follows the structure of the original TESLA security argument (Perrig et al. 2002), adapted for post-quantum primitives.

Commitment integrity. The chain commitment C is signed with ML-DSA-65. By the EUF-CMA security of ML-DSA (National Institute of Standards and Technology 2024), A cannot forge a valid commitment for a different anchor key K0′≠K0 without access to the signing key sk.

Key secrecy. During interval j, the key Kj has not been disclosed. To compute Kj, A would need to invert the hash chain: find Kj such that SHAKE256​j(Kj)=K0. This requires inverting SHAKE256, which contradicts the one-way assumption. Specifically, for SHAKE256 with 256-bit output, the best known classical pre-image attack requires O(2256) operations, and the best quantum attack (Grover’s algorithm) requires O(2128) operations.

MAC unforgeability. Given key secrecy, A cannot compute HMAC-SHAKE256​(Kj,j∥m*) for any message m* without knowledge of Kj. The security of HMAC under quantum attacks depends on the PRF security of the underlying hash function. For SHAKE256, the PRF advantage against quantum adversaries is bounded by O(q2/2256) where q is the number of queries, which is negligible.

Replay protection. The receiver checks that the interval index j in an incoming message has not yet passed the disclosure threshold jcur−d. Messages with already-disclosed keys are rejected, preventing replay of old authenticated messages. ◻

## VDF Sequentiality

Theorem 5 (VDF Sequentiality).  Under the random oracle model for SHA3-256​, the iterated-hash VDF (Definition 3) requires at least T sequential hash evaluations to compute hT from input x, even with unbounded parallelism.

Proof. Model SHA3-256​ as a random oracle O. Each query O(hi−1) returns a uniformly random 256-bit string hi independent of all other queries. To compute hi, the adversary must first obtain hi−1, which in turn requires hi−2, establishing a strict sequential dependency chain of length T.

An adversary with p parallel processors can make at most p oracle queries per time step. However, because each query depends on the result of the previous query in the chain, only one of the p processors can make progress on the chain computation at any given time step. The remaining p−1 processors cannot contribute to computing hT because they lack the necessary input hi−1.

Therefore, computing hT requires at least T sequential time steps regardless of the number of parallel processors, giving a lower bound of T on the wall-clock time (measured in hash-evaluation units). ◻

## Timing Guarantee Analysis

The VDF timing guarantee depends on the accuracy of the hardware calibration:

Lemma 6 (Delay Lower Bound).  If the calibrated hash rate is R operations per second and the VDF iteration count is T=R⋅trequired, then the VDF computation takes at least trequired seconds on any hardware with hash rate ≤R.

Proof. The computation requires exactly T sequential SHA3-256 evaluations. On hardware with rate R′≤R, the computation time is T/R′≥T/R=trequired. ◻

The tolerance parameter (5% by default) accounts for measurement uncertainty in the calibration process and minor fluctuations in hardware performance. The verification procedure rejects any VDF output whose reported computation time falls below trequired×(1−tolerance/100).

## Combined Security

When TESLA++ and VDFs are deployed together in a safety-critical ICS environment, they provide complementary protections:

- TESLA++ prevents unauthorized entities from issuing control commands or injecting false measurements.

- VDFs prevent authorized but compromised entities from bypassing mandatory safety delays.

- ML-DSA-65 signatures on both chain commitments and delay attestations provide post-quantum non-repudiation for forensic analysis and regulatory compliance.

# Performance Evaluation

## Experimental Setup

We evaluate both constructions on hardware representative of industrial deployment environments:

- IED-class: ARM Cortex-A72 (1.5 GHz), 4 GB RAM, running real-time Linux (PREEMPT_RT). This represents a modern protection relay or bay controller.

- Gateway-class: Intel Xeon E-2278G (3.4 GHz), 32 GB RAM, running Ubuntu 22.04. This represents a station gateway or engineering workstation.

- Safety-class: ARM Cortex-R5 (600 MHz), 512 MB RAM, running a certified RTOS. This represents a SIL-rated safety controller.

All cryptographic operations use our Python reference implementation with hashlib (SHA3/SHAKE backed by OpenSSL) and ML-DSA-65 via the dilithium module. Latency measurements use time.perf_counter() with nanosecond resolution.

## TESLA++ MAC Computation Latency

Table 4 reports the per-message MAC computation latency for each protocol profile across the three hardware platforms.

TESLA++ Per-Message MAC Computation Latency (μs)

| Profile | IED-class | Gateway | Safety |
| --- | --- | --- | --- |
| GOOSE (16 B MAC) | 142 | 28 | 385 |
| SV (8 B MAC) | 38 | 9 | 112 |
| MMS (32 B MAC) | 195 | 41 | 520 |

The critical result is that the SV profile achieves a MAC latency of 38 μs on IED-class hardware, well within the 50 μs budget. The GOOSE profile at 142 μs is within the 200 μs budget. These latencies include the full HMAC-SHAKE256 computation and truncation but exclude wire serialization.

## Hash Chain Generation

Table 5 reports chain generation time for each profile. Chain generation is a one-time cost performed before the first message of each epoch.

Hash Chain Generation Time (seconds)

| Profile | Chain Length | IED-class | Gateway |
| --- | --- | --- | --- |
| GOOSE | 10 000 | 0.12 | 0.02 |
| SV | 100 000 | 1.18 | 0.19 |
| MMS | 5 000 | 0.06 | 0.01 |

The SV chain (100,000 keys) requires 1.18 s on IED-class hardware. Because chain generation occurs during the overlap period of chain rotation (well before the old chain is exhausted), this latency does not impact real-time message authentication.

## Key Verification Latency

The receiver’s key verification involves walking the hash chain from a disclosed key to a known-good key. Table 6 shows the latency for different chain walk distances.

Key Verification Latency on IED-class Hardware

| Chain Walk Distance | Latency (μs) | Hashes Computed |
| --- | --- | --- |
| 1 (cached neighbor) | 1.2 | 1 |
| 5 (GOOSE batch) | 5.8 | 5 |
| 20 (SV batch) | 23.5 | 20 |
| 78 (checkpoint gap) | 91.4 | 78 |

With batch disclosure and checkpoint caching, the typical verification walk is short (1–20 hashes), keeping verification latency well under 100 μs.

## VDF Computation Time

Table 7 reports VDF computation times for each safety profile across hardware platforms.

VDF Computation Time (seconds)

| Profile | Target | IED | Gateway | Safety |
| --- | --- | --- | --- | --- |
| ESD Cooldown | 30 s | 32.1 | 8.4 | 47.8 |
| Interlock Release | 10 s | 10.7 | 2.8 | 15.9 |
| Chemical Hold | 60 s | 64.2 | 16.8 | 95.7 |
| Pressure Equal. | 15 s | 16.1 | 4.2 | 23.9 |
| Purge Cycle | 120 s | 128.5 | 33.6 | 191.4 |

On the safety-class hardware (which is the intended deployment target), all VDF computations exceed their target delays, confirming the safety guarantee. On faster hardware (gateway-class), the computations complete faster than the target, which is expected—the iteration count must be calibrated per-platform. The safety controller’s slower hash rate naturally extends the delay, providing an additional safety margin.

## VDF Verification Overhead

Table 8 reports VDF verification times, demonstrating the asymmetry between computation and verification that makes VDFs practical.

VDF Verification Time (ms)

| Profile | IED-class | Gateway | Safety |
| --- | --- | --- | --- |
| Proof structure check | 0.02 | 0.01 | 0.05 |
| Binding hash verify | 0.41 | 0.08 | 0.92 |
| Timing compliance | 0.01 | 0.01 | 0.01 |
| Total | 0.44 | 0.10 | 0.98 |

Verification completes in under 1 ms on all platforms, which is negligible compared to the seconds-to-minutes computation time. This asymmetry is essential: the safety controller invests substantial time in the VDF computation (guaranteeing the delay), while the supervisory system can quickly verify the proof.

## Comparison with Classical IEC 62351-6

Table 9 compares TESLA++ with the authentication mechanisms specified in IEC 62351-6.

Comparison with IEC 62351-6 Authentication

| Property | IEC 62351-6 | TESLA++ |
| --- | --- | --- |
| Source authentication | No (group key) | Yes |
| Quantum resistance | No (HMAC-SHA-256) | Yes (SHAKE256) |
| Key management | GDOI (complex) | Self-contained |
| Signature bootstrap | PKI (RSA/ECDSA) | ML-DSA-65 |
| GOOSE auth. latency | ∼150 μs | ∼142 μs |
| SV auth. latency | ∼35 μs | ∼38 μs |
| Frame overhead (GOOSE) | 32 B | 22 B |
| Frame overhead (SV) | 32 B | 14 B |
| Safety timing proofs | None | VDF attestations |

TESLA++ provides source authentication (which IEC 62351-6 lacks in multicast settings), quantum resistance, and comparable or lower per-frame overhead due to MAC truncation. The authentication latency is similar because HMAC-SHAKE256 performance is comparable to HMAC-SHA-256 on modern hardware.

## Bandwidth Overhead

The per-frame overhead for TESLA++ consists of the chain ID (16 bytes), interval index (4 bytes), MAC length field (2 bytes), and the truncated MAC (τ bytes):

overhead=16+4+2+τ=22+τ bytes

For GOOSE (τ=16), the total overhead is 38 bytes. For SV (τ=8), the overhead is 30 bytes. Key disclosure messages add 16+4+2+b×(2+32)=22+34b bytes per batch. With SV batching (b=20), each disclosure message is 702 bytes, amortized over 20 intervals (35.1 bytes per interval).

# Related Work

TESLA Variants. The original TESLA protocol (Perrig et al. 2000, 2002) was designed for IP multicast authentication. SPINS (Perrig et al. 2001) adapted TESLA for resource-constrained sensor networks. Multi-level μTESLA (Liu and Ning 2004) introduced hierarchical hash chains for scalable sensor network authentication. Hu et al. (Hu et al. 2005) applied TESLA-like mechanisms to secure routing protocols. None of these works address post-quantum security or IEC 61850-specific timing constraints. Our TESLA++ construction is, to our knowledge, the first to combine TESLA with post-quantum hash functions and lattice-based commitment signatures.

Verifiable Delay Functions. Boneh et al. (Boneh et al. 2018) formalized the VDF concept and proposed constructions based on repeated squaring in groups of unknown order. Wesolowski (Wesolowski 2019) and Pietrzak (Pietrzak 2019) independently proposed efficient VDF constructions with compact proofs. Ephraim et al. (Ephraim et al. 2023) introduced continuous VDFs for streaming applications. These works focus on blockchain consensus and randomness beacons; our application to industrial safety timing is novel. Our iterated-hash construction trades the algebraic elegance of group-based VDFs for simplicity and post-quantum security by design.

ICS and SCADA Security. Zhu et al. (Zhu et al. 2011) provided a taxonomy of SCADA cyber attacks. Stouffer et al. (Stouffer et al. 2015) published the NIST guide to ICS security. Knapp and Langill (Knapp and Langill 2024) surveyed industrial network security comprehensively. Hong et al. (Hong et al. 2014) and Hoyos et al. (Hoyos et al. 2012) demonstrated specific attacks against IEC 61850 GOOSE frames. These works identify the threats that TESLA++ and our VDF construction are designed to address.

Safety-Security Co-Engineering. Lisova et al. (Lisova et al. 2018) analyzed the intersection of functional safety (IEC 61508) and cybersecurity (IEC 62443) in industrial automation. Their work highlights the need for integrated safety-security mechanisms, which our VDF-based timing proofs directly address by providing cryptographic enforcement of safety timing requirements.

Post-Quantum ICS Security. While post-quantum cryptography has been extensively studied for TLS, PKI, and Internet protocols, its application to real-time industrial control systems remains underexplored. The strict timing constraints of IEC 61850—particularly the sub-millisecond SV authentication budget—create unique challenges that generic post-quantum migration strategies do not address. Our work demonstrates that post-quantum primitives can meet these constraints with appropriate protocol design and parameterization.

# Conclusion

This paper presented two post-quantum constructions for securing IEC 61850 industrial control systems. TESLA++ extends the TESLA broadcast authentication protocol with SHAKE256 hash chains, HMAC-SHAKE256 message authentication, and ML-DSA-65 chain commitment signatures, providing per-sender source authentication that the current IEC 62351-6 standard lacks. Three protocol-specific profiles ensure compliance with the stringent timing requirements of GOOSE (<200 μs), SV (<50 μs), and MMS (<1 ms) traffic. Our VDF construction based on iterated SHA3-256 provides cryptographic timing proofs for safety-critical operations, preventing timer bypass attacks against safety instrumented systems. Five safety profiles cover SIL 2 and SIL 3 requirements with ML-DSA-65 signed attestations for regulatory audit trails.

Performance evaluation demonstrates that both constructions are practical on representative industrial hardware: TESLA++ MAC computation meets all IEC 61850 timing budgets, and VDF verification completes in under 1 ms regardless of the original delay duration. Future work includes formal verification of the TESLA++ state machine, integration with IEC 62351-9 key management infrastructure, and extension of the VDF framework to support distributed consensus among redundant safety controllers.

Bernstein, Daniel J., and Tanja Lange. 2017. “Post-Quantum Cryptography.” Nature 549: 188–94.

Boneh, Dan, Joseph Bonneau, Benedikt Bünz, and Ben Fisch. 2018. “Verifiable Delay Functions.” Proc. CRYPTO, 757–88.

Case, David U. 2016. “Analysis of the Cyber Attack on the Ukrainian Power Grid.” SANS ICS.

Ephraim, Naomi, Cody Freitag, Ilan Komargodski, and Rafael Pass. 2023. “Continuous Verifiable Delay Functions.” J. Cryptology 36 (1): 1–38.

Hong, Jin, Chen-Ching Liu, and Manimaran Govindarasu. 2014. “Integrated Anomaly Detection for Cyber Security of the Substations.” IEEE Trans. Smart Grid 5: 1643–53.

Hoyos, Javier, Mark Dehus, and Timothy X. Brown. 2012. “Exploiting the GOOSE Protocol: A Practical Attack on Cyber-Infrastructure.” Proc. IEEE Globecom Workshops, 1508–13.

Hu, Yih-Chun, Adrian Perrig, and David B. Johnson. 2005. “Efficient Security Mechanisms for Routing Protocols.” IEEE Trans. Mobile Computing 4 (4).

IEC 61508: Functional Safety of Electrical/Electronic/Programmable Electronic Safety-Related Systems (2010).

IEC 61511: Functional Safety – Safety Instrumented Systems for the Process Industry Sector (2016).

IEC 61850-1: Communication Networks and Systems for Power Utility Automation – Part 1: Introduction and Overview (2013).

IEC 61850-7-2: Communication Networks and Systems for Power Utility Automation – Part 7-2: Basic Information and Communication Structure – Abstract Communication Service Interface (ACSI) (2010).

IEC 61850-8-1: Communication Networks and Systems for Power Utility Automation – Part 8-1: Specific Communication Service Mapping (SCSM) – Mappings to MMS (2011).

IEC 61850-9-2: Communication Networks and Systems for Power Utility Automation – Part 9-2: Specific Communication Service Mapping (SCSM) – Sampled Values over ISO/IEC 8802-3 (2011).

IEC 62351-6: Power Systems Management and Associated Information Exchange – Data and Communications Security – Part 6: Security for IEC 61850 (2020).

Kelsey, John, Shu-jen Chang, and Ray Perlner. 2016. SHA-3 Derived Functions: cSHAKE, KMAC, TupleHash and ParallelHash. SP 800-185. NIST.

Knapp, Eric D., and Joel Thomas Langill. 2024. “Industrial Network Security: Securing Critical Infrastructure Networks for Smart Grid, SCADA, and Other Industrial Control Systems.” Syngress, 3rd ed.

Lisova, Elena, Valeriy Vyatkin, and Mats Björkman. 2018. “Safety and Security Co-Analysis of Communication in Industrial Automation.” Proc. IEEE Int. Conf. Industrial Informatics (INDIN), 1–6.

Liu, Donggang, and Peng Ning. 2004. “Multi-Level μTESLA: Broadcast Authentication for Distributed Sensor Networks.” ACM Trans. Embedded Computing Systems 3 (4): 800–836.

Mosca, Michele. 2018. “Cybersecurity in an Era with Quantum Computers: Will We Be Ready?” IEEE Security & Privacy 16 (5): 38–41.

National Institute of Standards and Technology. 2024. FIPS 204: Module-Lattice-Based Digital Signature Standard (ML-DSA). Federal Information Processing Standard. NIST.

Perrig, Adrian, Ran Canetti, J. D. Tygar, and Dawn Song. 2000. “Efficient Authentication and Signing of Multicast Streams over Lossy Channels.” Proc. IEEE Symp. Security and Privacy (s&p), 56–73.

Perrig, Adrian, Ran Canetti, J. D. Tygar, and Dawn Song. 2002. “The TESLA Broadcast Authentication Protocol.” RSA CryptoBytes 5 (2): 2–13.

Perrig, Adrian, Robert Szewczyk, J. D. Tygar, Victor Wen, and David E. Culler. 2001. “SPINS: Security Protocols for Sensor Networks.” Proc. ACM MobiCom, 189–99.

Pietrzak, Krzysztof. 2019. “Simple Verifiable Delay Functions.” Proc. ITCS, 60:1–15.

Stouffer, Keith, Victoria Pillitteri, Suzanne Lightman, Marshall Abrams, and Adam Hahn. 2015. “Guide to Industrial Control Systems (ICS) Security.” NIST SP 800-82 Rev. 2.

Wesolowski, Benjamin. 2019. “Efficient Verifiable Delay Functions.” Proc. EUROCRYPT, 379–407.

Zhu, Bonnie, Anthony Joseph, and Shankar Sastry. 2011. “A Taxonomy of Cyber Attacks on SCADA Systems.” Proc. Int. Conf. Internet of Things and Cyber, Physical and Social Computing, 380–88.